TRANSPORT AND TELECOMMUNICATION INSTITUTE

ENGINEERING FACULTY

Exam Practical Task

Course: Computer Systems Structure

Theme: RISC Processor: AVR ASM

Student: Igors Oļeiņikovs

Student code: 93642

Group: 4501BTA

Riga

2026

CONTENTS

Contents

1.	TASK DESCRIPTION	3

2.	PIN MAPPING	3

3.	IMPLEMENTATION SUMMARY	4

4.	PROGRAM DESIGN AND LOGIC	5

5.	KEY MECHANISM	9

# TASK DESCRIPTION

The task requires creating a setup in the Wokwi simulation platform and writing a program in AVR Assembly for the Arduino Uno (ATmega328P). The program must drive five LEDs and one seven segment display, launched by a push button, without using any Arduino C/C++ functions - only direct AVR register and port manipulation

Seven segment display must show the numeric digits of the student code one by one, each for approximately one second. While a digit is shown, the LEDs must indicate whether the digit is odd (LEDs 1, 3, 5 ON) or even (LEDs 2, 4 ON). After all digits displayed, running light effect moves from LED1 to LED5 (0.5 s each) with the display blank. The system then returns to the initial alternating LED pattern and waits for the next button press.

Student code digits

Student code: 93642

Numeric sequence:  9 > 3 > 6 > 4 > 2

| Step | Seven segment display | Digit type | LED output |
| --- | --- | --- | --- |
| 1 | 9 | Odd | LEDs 1, 3, and 5 ON |
| 2 | 3 | Odd | LEDs 1, 3, and 5 ON |
| 3 | 6 | Even | LEDs 2 and 4 ON |
| 4 | 4 | Even | LEDs 2 and 4 ON |
| 5 | 2 | Even | LEDs 2 and 4 ON |

# PIN MAPPING

The following table shows which AVR port and bit is connected to each hardware component. All five LEDs are on Port B, the seven-segment segments A - F are on Port C, segment G is on Port D, and the button uses Port D with the internal pull-up resistor enabled.

| Component | AVR port / bit | Arduino pin | Direction |
| --- | --- | --- | --- |
| LED1 (odd) | PB5 | 13 | Output |
| LED2 (even) | PB4 | 12 | Output |
| LED3 (odd) | PB3 | 11 | Output |
| LED4 (even) | PB2 | 10 | Output |
| LED5 (odd) | PB1 | 9 | Output |
| 7-seg A | PC0 | A0 | Output |
| 7-seg B | PC1 | A1 | Output |
| 7-seg C | PC2 | A2 | Output |
| 7-seg D | PC3 | A3 | Output |
| 7-seg E | PC4 | A4 | Output |
| 7-seg F | PC5 | A5 | Output |
| 7-seg G | PD3 | 3 | Output |
| Button | PD2 | 2 | Input (pull-up) |
| 7-seg COM | GND | — | Common cathode |

Fig.1 Wokwi circuit

# IMPLEMENTATION SUMMARY

Five LEDs are connected to Arduino Uno pins 13 down to 9 through separate 1 kOhm resistors. These pins correspond to PB5, PB4, PB3, PB2 and PB1 on the ATmega328P. A common cathode seven segment display is driven from Port C (PC0 - PC5 for segments A - F) and PD3 for segment G. The common pin is connected to ground through a 1 kOhm resistor. A push button is connected to PD2 (pin 2) with the internal pull-up enabled, so the pin reads high when the button is released and low when it is pressed.

The program is organized into clearly separated sections: port initialization, the initial alternating LED state, button waiting with debounce, the student code display phase with odd/even LED selection, the running light finish phase and the return to the waiting state. Two lookup tables stored in program memory are read with the lpm instruction: one holds the seven segment patterns for digits 0 - 9 and the other holds the numeric digits of the student code in order.

# PROGRAM DESIGN AND LOGIC

Port initialization

DDRB = 0x3E configures PB5 - PB1 as LED outputs.

DDRC = 0x3F configures PC5 - PC0 as outputs for segments A - F.

PD3 is set as an output for segment G.

PD2 is kept as an input and PORTD bit 2 is set high to enable the internal pull-up resistor on the button pin.

Initial state

The seven segment display is blank (all segments off).

PORTB is set to 0x2A, which turns on PB5, PB3 and PB1 - the alternating pattern LED1 ON, LED2 OFF, LED3 ON, LED4 OFF, LED5 ON.

The program polls PD2 until the button is pressed.

Button launch

The program waits for PD2 to go low (press), applies a 20 ms debounce delay, and confirms the press is still valid.

It then waits for PD2 to return high (release) and applies another 20 ms debounce delay. This prevents a single long press from restarting the sequence multiple times.

Student code display phase

A counter (r20) is set to 5, the number of digits in the student code.

The Z register points to code_table in program memory. Each iteration loads the next digit with lpm r21, Z+.

display_digit looks up the seven segment pattern in seg_table and writes the lower six bits to PORTC and bit 6 to PD3. The subroutine saves and restores Z internally so the caller's pointer is preserved

The digit's least significant bit is tested with andi r22, 0x01: if set, the digit is odd and PORTB = 0x2A (LEDs 1, 3, 5) - if clear, the digit is even and PORTB = 0x14 (LEDs 2, 4).

A ~1 sec delay is applied before moving to the next digit.

Running light finish phase

The display is blank and a counter (r18) is set to 5.

A single bit in r17 starts at 0x20 (LED1 / PB5) and is shifted right with lsr after each 0.5 s delay, lightning LED1 through LED5 in turn.

After LED5 the program jumps back to initial state, restoring the alternating LED pattern and waiting for next button press

#define __SFR_OFFSET 0

#include "avr/io.h"

.global main

; LED patterns on PORTB bits PB5..PB1

; LED1=PB5, LED2=PB4, LED3=PB3, LED4=PB2, LED5=PB1

.equ LED_ODD,    0x2A   ; LEDs 1,3,5 ON

.equ LED_EVEN,   0x14   ; LEDs 2,4 ON

.equ LED_RUN_1,  0x20   ; running-light start at LED1

.equ DIGIT_COUNT, 5

main:

; Port initialization

ldi   r16, 0x3E

out   DDRB, r16

ldi   r16, 0x3F

out   DDRC, r16

sbi   DDRD, 3

cbi   DDRD, 2

sbi   PORTD, 2

; Initial state

initial_state:

call  display_blank

ldi   r16, LED_ODD

out   PORTB, r16

; Button waiting

wait_press:

sbis  PIND, 2

rjmp  button_pressed

rjmp  wait_press

button_pressed:

ldi   r25, hi8(20)

ldi   r24, lo8(20)

call  delay_ms

sbis  PIND, 2

rjmp  wait_release

rjmp  wait_press

wait_release:

sbis  PIND, 2

rjmp  wait_release

ldi   r25, hi8(20)

ldi   r24, lo8(20)

call  delay_ms

; Student code display phase

ldi   r20, DIGIT_COUNT

ldi   ZH, hi8(code_table)

ldi   ZL, lo8(code_table)

display_loop:

lpm   r21, Z+

call  display_digit

mov   r22, r21

andi  r22, 0x01

breq  even_digit

odd_digit:

ldi   r16, LED_ODD

out   PORTB, r16

rjmp  digit_hold

even_digit:

ldi   r16, LED_EVEN

out   PORTB, r16

digit_hold:

ldi   r25, hi8(1000)

ldi   r24, lo8(1000)

call  delay_ms

dec   r20

brne  display_loop

; Running light finish phase

call  display_blank

ldi   r18, DIGIT_COUNT

ldi   r17, LED_RUN_1

running_light:

out   PORTB, r17

ldi   r25, hi8(500)

ldi   r24, lo8(500)

call  delay_ms

lsr   r17

dec   r18

brne  running_light

; Return to waiting state

rjmp  initial_state

; Subroutine: display_blank

; Turns off every segment of display

display_blank:

ldi   r16, 0x00

out   PORTC, r16

cbi   PORTD, 3

ret

; Subroutine: display_digit — show r21 (0-9) on 7-seg

; Saves and restores Z so callers Z is preserved

display_digit:

push  r30

push  r31

ldi   ZH, hi8(seg_table)

ldi   ZL, lo8(seg_table)

add   ZL, r21

clr   r19

adc   ZH, r19

lpm   r23, Z

mov   r19, r23

andi  r19, 0x3F

out   PORTC, r19

sbrc  r23, 6

rjmp  seg_g_on

cbi   PORTD, 3

pop   r31

pop   r30

ret

seg_g_on:

sbi   PORTD, 3

pop   r31

pop   r30

ret

; Subroutine: delay_ms

delay_ms:

ldi   r29, hi8(4000)

ldi   r28, lo8(4000)

delay_inner:

sbiw  r28, 1

brne  delay_inner

sbiw  r24, 1

brne  delay_ms

ret

; Seven-segment patterns for digits 0..9

; Bit mapping: bit0=A bit1=B bit2=C bit3=D bit4=E bit5=F bit6=G

seg_table:

.byte 0x3F, 0x06, 0x5B, 0x4F, 0x66, 0x6D, 0x7D, 0x07, 0x7F, 0x6F

code_table:

.byte 9, 3, 6, 4, 2

# KEY MECHANISM

How the button is checked

The button is connected to PD2 with the internal pull-up enabled, so the pin is HIGH when the button is released and LOW when pressed. The program uses sbis PIND, 2 to skip the next instruction when the pin is HIGH. A 20 ms debounce delay is applied both after detecting a press and after detecting a release. The program first waits for a press, then waits for a release, ensuring the main sequence launches exactly once per button cycle.

How the odd/even LED pattern is selected

After loading a digit from code_table, the program copies it into r22 and executes andi r22, 0x01 to isolate the least significant bit. If the bit is 1 the digit is odd and PORTB is loaded with 0x2A (LEDs 1, 3, 5 ON). If the bit is 0 the digit is even and PORTB is loaded with 0x14 (LEDs 2, 4 ON). The breq instruction branches to the appropriate pattern.

How the delay timing is created

The delay_ms subroutine accepts a 16-bit millisecond count in r25:r24. For each millisecond it runs an inner loop of approximately 4000 iterations, each consuming four clock cycles (sbiw + brne). At 16 MHz, four cycles x 4000 = 16000 cycles = approx 1 ms. The outer loop decrements r25:r24. For the digit display phase, a value of 1000 is passed for ~1 second per digit. For the running light phase, a value of 500 is passed for ~0.5 second per step. The Y register (r29:r28) is used for the inner loop to avoid conflicts with the Z register.

How the seven segment digit patterns are selected

A lookup table seg_table in program memory stores one byte per digit (0-9). Each byte is a 7-bit pattern where bit 0 = segment A, bit 1 = B, bit 2 = C, bit 3 = D, bit 4 = E, bit 5 = F, and bit 6 = G. The display_digit subroutine calculates the address as seg_table + r21, reads the byte with lpm, writes bits 0-5 to PORTC (segments A-F), and tests bit 6 to control PD3 (segment G). The subroutine preserves the caller's Z register by pushing it at entry and popping it on both return paths.

How the code table pointer is preserved

The display_digit subroutine internally modifies the Z register to access seg_table. To prevent this from corrupting the code_table pointer used by display_loop, display_digit saves r30 (ZL) and r31 (ZH) on the stack at entry and restores them before returning. Both the segment G-on and segment G-off return path include the pop instructions, ensuring correct Z restoration regardless of which path is taken. This allows display_loop to iterate through all five student code digits without any additional save/restore logic in the caller.

Video demonstration  attached

Project saved at https://wokwi.com/projects/467947593630873601