Assessment Report

Module Details

| Module Code | UFCFCW-24-0 |  |
| --- | --- | --- |
| Module Title | Computer System Structure part 1 |  |
| Lecturer | Vasil Peravoshchykau |  |
| Students Name: |  |  |
| Year | 2024/2025 |  |
| Semester | I Semester |  |
| Assessment Topic | RISC Processor: AVR ASM |  |
| Assignment | Delivered/Not Delivered | Grade |
| Exercise 1 |  | /20 |
| Exercise 2 |  | /20 |
| Exercise 3 |  | /20 |
| Exercise 4 |  | /20 |
|  | Total: | /100 |

Dates

| Submission Date |  |
| --- | --- |
| Submission Deadline |  |
| Submission Place | TSI LMS (e.tsi.lv) |
| Submission Time | 17:00 |
| Submission Notes | The assessments that were submitted after the deadline will be graded with a lower grades. |

Feedback

| Feedback provision will be | The feedback will be provided using TSI LMS |
| --- | --- |

YOU SHOULD NOT EXPECT TO GET ANY REMINDERS FROM TUTORS ABOUT ANY OF THESE

RESPONSIBILITIES OR DEADLINES

Exercise 1: Setting up the device connection

Objective: Create the test setup, upload the example program.

Required Equipment:

- Breadboard – 1 pcs

- Arduino UNO board  – 1 pcs

- LED – 1 pcs

- 1kOm resistor  – 1 pcs

- Jumper wires – 1 set

Tasks:

- Create a test setup. The components required:

  - Breadboard

  - Arduino UNO board

  - 1 LED connected between Pin8 and GND using 1kOhm resistor.

    - NOTE: you MUST use 1kOhm resistor to connect LED to Arduino board

- Explore the program running and the sest setup on the simulation: https://wokwi.com/projects/290348681199092237 .

- Upload the example program to the Arduino UNO board

- Write a brief report of the process. Take a video or photo of the blinking LED

Example program:

; Blink LED on PB5(Arduino Uno pin 13) http://forum.arduino.cc/index.php?topic=159572#msg1194604

#define __SFR_OFFSET 0

#include "avr/io.h"

.global main

main:

sbi DDRB, 5 ; Set PB5 as output

blink:

sbi PINB, 5 ; Toggle PINB

ldi r25, hi8(1000)

ldi r24, lo8(1000)

call delay_ms

jmp blink

delay_ms:

; Delay about (r25:r24)*ms. Clobbers r30, and r31. One millisecond is about 16000 cycles at 16MHz. The inner loop takes 4 cycles, so we repeat it 3000 times

ldi r31, hi8(4000)

ldi r30, lo8(4000)

1:

sbiw r30, 1

brne 1b

sbiw r24, 1

brne delay_ms

ret

REPORT (you may extend the report as needed beyond this page)

Exercise 2: Running lights on Arduino UNO board using AVR Assembler

Objective: Review the example program from the Exercise 1 and make the following changes of it’s behaviour.

Required Equipment:

- Breadboard – 1 pcs

- Arduino UNO board  – 1 pcs

- LEDs – 5 pcs

- 1kOm resistors  – 5 pcs

- USB cable – 1 pcs

- Jumper wires – 1 set

Tasks:

- Connect additional LEDs to PIN13 to PIN9 (don’t forget to use 1kOm resistors!)

- Create the effect of running lights from one LED to another.

- Use the same time pause between all LED changes.

- Write a report detailing:

  - Your program's design and logic.

  - Challenges faced and strategies used.

  - The explanation of how the time pause could be changed.

- Save your program code and put it into your report. The code should be clearly commented, explaining each instruction

- Make the screenshots or video footage.

REPORT (you may extend the report as needed beyond this page)

Exercise 3:  LEDS and Buttons control on Arduino UNO using AVR Assembler

Objective: Create a program that is controlling LEDs and buttons.

Required Equipment:

- Breadboard – 1 pcs

- Arduino UNO board  – 1 pcs

- LEDs – 5 pcs

- 1kOm resistors  – 5 pcs

- Button – 1 pcs

- USB cable – 1 pcs

- Jumper wires – 1 set

Tasks:

- Design a program that uses the following:

  - A button is connected to the port D (any pin).

  - LEDs are connected to the port B (Don’t forget to use 1kOhm resistor)

  - At the beginning all odd LEDs are turned on and all even LEDs are turned off.

  - If the button is pressed, all LEDs change their state to opposite (odds are off and evens are on and vice versa)

- Write a report detailing:

  - Your program's design and logic.

  - Challenges faced and strategies used.

- Save your program code and put it into your report. The code should be clearly commented, explaining each instruction

- Make a video or photos of the behaviour of your Arduino board setup.

REPORT (you may extend the report as needed beyond this page)

Exercise 4:  7-segment display with Arduino UNO using AVR Assembler

Objective: Create a program that is controlling 7-segment display.

Required Equipment:

- Breadboard – 1 pcs

- Arduino UNO board  – 1 pcs

- 7-segment display – 1 pcs

- 1kOm resistor  – 1 pcs

- Button – 1 pcs

- Active buzzer – 1 pcs

- USB cable – 1 pcs

- Jumper wires – 1 set

Tasks:

- Design a program that uses the following:

  - A button is connected to the port D (any pin).

  - An active buzzer is connected to the Port D (any other pin)

  - 7-segment display segments are connected to Port B (Don’t forget to use 1kOhm resistor)

  - At the beginning, the display shows digit 9.

  - Each second the display counts down (9-8-7-6-…-0)

  - If the digit is 0 then the active buzzer is enabled so the sound is generated.

  - If the button is pressed, the display is reset to 9 again and starting from the beginning.

- Write a report detailing:

  - Your program's design and logic.

  - Challenges faced and strategies used.

- Save your program code and put it into your report. The code should be clearly commented, explaining each instruction

- Make a video or photos of the behaviour of your Arduino board setup.

REPORT (you may extend the report as needed beyond this page)