TRANSPORT AND TELECOMMUNICATION INSTITUTE

ENGINEERING FACULTY

Practical work N1

Course: Computer Systems Structure

Theme: The Von Neumann Machine

Student: Igors Oļeiņikovs

Student code: 93642

Group: 4501BTA

Riga

2026

CONTENTS

1.	EXERCISE 1	3

2.	EXERCISE 2	5

3.	EXERCISE 3	14

4.	EXERCISE 4	19

5.	CONCLUSION	26

# EXERCISE 1

Exercise 1: Understanding the vnmsim Interface

Objective: Familiarize with the vnmsim platform and identify components of the Von Neumann architecture.

Fig. 1 Initial vnmsim interface

Main elements visible in the simulator

| Interface element | Role in vnmsim | Von Neumann architecture meaning |
| --- | --- | --- |
| Top menu (1) | Gives access to New project, Open from file, Save to file, Samples, Statistics, Help, and Settings | These controls do not belong to the theoretical machine itself; they are the environment used to load programs and inspect execution |
| RAM editor on the left (2) | Text area where each memory line contains an instruction or numeric cell | Represents main memory, where program instructions and data live in the same address space |
| Variable table (X, Y, Z, W, T0/T1...) (3) | Editable values used by the current program | Represents data stored in RAM cells; T variables are temporary memory locations |
| Add T variable button (4) | Creates more temporary variables | Extends the available memory cells for intermediate storage |
| IR (Instruction Register) (5) | Shows the current decoded instruction and operand | Holds the instruction currently being executed |
| Decoder (6) | Indicates the control unit stage that interprets the current instruction | Represents the control unit that decides how hardware reacts to the opcode |
| ALU (7) | Shows the arithmetic operation and operands currently in use | Executes arithmetic and logic operations |
| ACC (Accumulator) (8) | Stores the current working value | Central working register used by this machine for arithmetic |
| PC (Program Counter) (9) | Shows the address of the next instruction | Points to the next memory location to fetch |
| Data bus (10) | Visual connection between memory, IR, ALU, and ACC | Carries data and instructions between components |
| Address bus (11) | Visual connection used with the PC and RAM | Carries memory addresses |
| RAM block on the diagram (12) | Abstract diagrammatic memory block | Highlights that both code and data share the same memory system |
| Execution controls (13) | Run loop, Single step, Single iteration, Pause, Stop | Let the user observe the fetch-decode-execute cycle |

Correlation with the Von Neumann model

The simulator follows the classic Von Neumann idea that instructions and data are stored in the same memory. The program text written in the RAM editor and the variables on the left are both part of the machine memory. The PC selects the next instruction, the IR stores the fetched instruction, the Decoder interprets it, and the ALU plus ACC carry out the computation. The buses drawn on the right make the data flow explicit, which is useful for understanding the fetch-decode-execute cycle rather than only seeing final numeric results.

# EXERCISE 2

Exercise 2: Exploring Pre-existing Programs

The vnmsim sample library contains 11 example programs. Most of them work correctly and are useful to study how a very small instruction set can still implement non-trivial behavior. The only problematic built-in sample is the one titled Modulo, whose code does not compute a modulo and instead behaves like a comparison routine.

Fig. 2 Sample programs

Sample programs analysis

| Sample | Purpose | Initial state | Final state | Works as intended? | Explanation |
| --- | --- | --- | --- | --- | --- |
| Addition | Compute X + Y = Z | X=2, Y=2, Z=0, W=0 | Z=4 | Yes | LOD X loads the accumulator, ADD Y adds the second operand, and STO Z saves the result. |
| Subtraction | Compute X - Y = Z | X=3, Y=2, Z=0, W=0 | Z=1 | Yes | Same structure as the addition sample, but using SUB Y. |
| Multiplication | Compute X * Y = Z | X=6, Y=7, Z=0, W=0 | Z=42 | Yes | Shows direct use of the MUL instruction with no branching. |
| Division | Compute X / Y = Z | X=12, Y=3, Z=0, W=0 | Z=4 | Yes | Minimal arithmetic example for DIV. |
| Basics | Compute X[Y]Z = W, where Y selects +, -, *, or / | X=3, Y=0, Z=2, W=0 | W=5 | Yes | The program repeatedly subtracts 1 from Y to detect whether the selected operation is 0, 1, 2, or 3, then jumps to the correct arithmetic branch. |
| Power | Compute X^Y = Z | X=3, Y=2, Z=0, W=0 | Y=0, Z=9 | Yes | Initializes Z to 1, then loops: multiply Z by X, decrement Y, repeat until Y=0. |
| Square root | Approximate sqrt(X) = Y | X=25, Y=0, Z=0, W=0 | Y=5, Z=0 | Yes | Uses an iterative method: initialize Y, keep a loop counter in Z, then update Y = (Y + X / Y) / 2 until the counter reaches zero. |
| Is even | Decide whether X is even and store the result in Z | X=4, Y=0, Z=0, W=0 | Y=4, Z=1 | Yes | Divides by two, reconstructs 2 * floor(X/2) into Y, then checks whether X - Y = 0. |
| Greater than | Decide whether X > Y and store the Boolean result in Z | X=3, Y=2, Z=0, W=0, T1..T4=0 | Z=1 | Yes | Because the machine has only JMZ, the sample simulates comparison by copying positive and negative counters into temporary variables and decrementing them until one side reaches zero first. |
| Is negative | Decide whether X < 0 and store the Boolean result in Z | X=-3, Y=0, Z=0, W=0, T1=0, T2=0 | Z=1, T2=3 | Yes | The program derives helper counters from X, then uses repeated subtraction and JMZ to distinguish negative from non-negative input. |
| Modulo | Title suggests X % Y = Z | X=3, Y=2, Z=0, W=0 | X=0, Y=0, Z=1 | No | The code repeatedly decrements X and Y together and sets Z to either 0 or 1. It behaves like a comparison routine, not a modulo implementation. The sample appears mislabeled or incomplete. |

Fig.3 Addition

Fig. 4 Subtraction

Fig.5 Multiplication

Fig. 6 Division

Fig. 7 Basics

Fig. 8 Power

Fig. 9 Square root

Fig. 10 Greather than

Fig. 11 Is even

Fig. 12 Is negative

Fig. 13 Modulo

General observations about the samples

1.	Straight arithmetic operations are very compact: LOD, one ALU instruction, STO, HLT.

2.	Branching is limited to JMZ, so every non-trivial decision is reduced to checking whether the accumulator is exactly zero.

3.	Because there is no indexed addressing over T1..T10, more advanced programs often rely on repeated subtraction and extra temporary variables.

4.	The sample set is useful because it shows both the strength and the limitation of accumulator-based programming: simple math is easy, but comparisons and selectors require creative control flow.

# EXERCISE 3

Exercise 3: Conditional swapping of two numbers

Requested behavior

•	Read two numbers from T1 and T2.

•	Read control value X.

•	If X = 0, do not swap.

•	If X = 1, swap T1 and T2.

•	For any other value, stop without changing the data.

Program design

The simulator only provides a zero-test jump (JMZ). Because of that, the cleanest way to detect

X = 1 is:

- Load X.

- If it is already zero, stop immediately.

- Subtract 1.

- If the result is zero, then the original value was 1, so run the swap.

- Otherwise halt.

The swap itself needs one temporary memory location, so T3 is used as auxiliary storage.

Program

LOD X // load the control value
JMZ 11 // X = 0 -> keep T1 and T2 unchanged
SUB #1 // ACC = X - 1
JMZ 5 // X = 1 -> execute the swap
HLT
LOD T1 // save the original first number
STO T3 // use T3 as temporary storage
LOD T2 // load the second number
STO T1 // copy T2 into T1
LOD T3 // recover the original T1 value
STO T2 // copy the original T1 into T2
HLT

Example test cases

| Case | Initial values | Expected final values | Result |
| --- | --- | --- | --- |
| Keep values | X=0, T0=0, T1=8, T2=3, T3=0 | T0=0, T1=8, T2=3, T3=0 | No swap |
| Swap values | X=1, T0=0, T1=8, T2=3, T3=0 | T0=0, T1=3, T2=8, T3=8 | Swap performed |
| Invalid selector | X=2, T0=0, T1=8, T2=3, T3=0 | T0=0, T1=8, T2=3, T3=0 | Safe halt |

Challenges and solution choices

The main limitation was the absence of a direct instruction such as JNZ, JGT, or CMP. The solution avoids unnecessary complexity by using two zero-tests in sequence. This is reliable, short, and easy to explain.

Fig.14 Before run (keep values)

Fig. 15 After run (keep values)

Fig. 16 Before run (swap values)

Fig. 17 After run (swap values)

Fig. 18 Invalid selector before run

Fig. 19 Invalid selector after run

# EXERCISE 4

Exercise 4: Conditional Addition of Numbers from an Array

Requested behavior

•	Store ten numbers in T1..T10.

•	If X = 0, sum all ten values.

•	If X = 1, sum the first five values.

•	If X = 2, sum the last five values.

•	If X has any other value, do nothing.

Program design

The result is stored in W.

Because vnmsim does not support indexed access to T1..T10, the most robust implementation does not use a counter-driven loop. Although JMZ can create loops (as seen in the Power and Square Root samples), vnmsim has no indirect addressing, so there is no way to access T1..T10 by a variable index. The additions are therefore unrolled explicitly. This matches the simulator's limitations and keeps the control flow easy to verify.

The selector logic is:

- If X = 0, jump to the branch that sums T1..T10.

- Otherwise subtract 1 and check whether the original value was 1.

- Otherwise subtract 1 again and check whether the original value was 2.

- If none of those branches match, halt without changing W.

Program code

LOD X // load the selector value
JMZ 7 // X = 0 -> sum all ten numbers
SUB #1 // ACC = X - 1
JMZ 19 // X = 1 -> sum the first five numbers
SUB #1 // ACC = X - 2
JMZ 26 // X = 2 -> sum the last five numbers
HLT
LOD T1 // start full-array sum
ADD T2 // add T2
ADD T3 // add T3
ADD T4 // add T4
ADD T5 // add T5
ADD T6 // add T6
ADD T7 // add T7
ADD T8 // add T8
ADD T9 // add T9
ADD T10 // add T10
STO W // store the total sum in W
HLT
LOD T1 // start first-half sum
ADD T2 // add T2
ADD T3 // add T3
ADD T4 // add T4
ADD T5 // add T5
STO W // store the sum of T1..T5 in W
HLT
LOD T6 // start second-half sum
ADD T7 // add T7
ADD T8 // add T8
ADD T9 // add T9
ADD T10 // add T10
STO W // store the sum of T6..T10 in W
HLT

Example test cases

For the following cases the input array is:

T1=1, T2=2, T3=3, T4=4, T5=5, T6=6, T7=7, T8=8, T9=9, T10=10

| Selector X | Required behavior | Expected result in W |
| --- | --- | --- |
| 0 | Sum all ten numbers | 55 |
| 1 | Sum first five numbers | 15 |
| 2 | Sum last five numbers | 40 |
| 9 | Do nothing | 0 |

Challenges and solution choices

The largest difficulty was the lack of indirect addressing. A normal loop over T1..T10 driven by an index variable is not possible in this simulator. JMZ jumps can create loops, but they can only reference fixed, named cells. Because there is no indirect addressing, there is no way to write LOD T[counter] where the address changes each iteration. The answer is therefore an unrolled program with three clearly separated branches. It is longer, but it is correct and easy to debug.

Fig. 20 Total sum (before run)

Fig. 21 Total sum (after run)

Fig. 22 Sum first five (before run)

Fig. 23 Sum first five (after run)

Fig. 24 Sum last five (before run)

Fig. 25 Sum last five (after run)

Fig. 26 Invalid selector (before run)

Fig. 27 Invalid selector (after run)

# CONCLUSION

This practical work shows that even a very small Von Neumann-style machine can solve meaningful problems, but simple high-level ideas such as comparison, selection, and array processing become much more manual when the instruction set only provides an accumulator and a jump-if-zero operation. That is what makes vnmsim useful for learning: it exposes the control flow and data movement that higher-level languages usually hide.